Differences in Orion and Pegasus RTLS

Orion:

General:
- Support Hardware version. Middle byte of the version displayed in Core is the hardware version
- Supports 4-byte device Id (2-bit device family identification + 30 bits device id)

UHF:  
- Location & Paging - 50 kbps;
- UHF Upgrade: 150 kbps (CC110 / CC1120 - All legacy devices) / 100 kbps (G4, Andromeda Tags).
- Supports 15 operating frequency
- Beacon transmitted at the middle of the beacon slot.
- Supports 12 frequencies for super sync  (108 unique slot + frequency)
- Number of OTA packets per duty cycle: 8
- Supports OnDemand upgrade
- G4 supports Listen before Talk feature during Ondemand upgrade
- Location Transmission once in every 1.5 secs

IR:
- IR Profile: 1.5, 3.0 and 6.0
- Receives 6 IR for every IR open, and option to configure IR TX in 6 IR Sub slots.
- Option to transmit IR in Regular, A/B profile
- No VW, VWH profiles
- Predefined & Custom power level and noise level configuration
- No Gradient Noise support.

- IR Sync available in Async mode.
- Support profile configuration for Async Mode IR.  (3.1 mode IR configuration)
- Tags will open of IR once every IR rate when tags is in active, and once in every 5 mins when Tag is in sleep.
- In Async Room change will be notified immediately
- Monitors in Async mode transmits Marker every 250ms.

LF: 
- Supports both PPM & Manchester mode LF transmission

Pegasus

General:
- No Hardware version.
- Supports 3 byte device Id (1 bit device family identification + 23 bits device id for Tags, 22 bit device id for monitors)

UHF: 
- Location & Paging - 76.8 kbps;
- UHF Upgrade: - 76.8 kbps;
- Supports 13 operating frequency
- Beacon transmitted at the beginning of the beacon slot.
- Supports 4 frequencies for super sync (36 unique slot + frequency)
- Number of OTA packets per duty cycle: 4
- No On demand upgrade support.
- G4 supports Listen before Talk feature during On demand upgrade
- Locaton Transmission once in every 3.0 secs

IR:
- IR Profile: 1.5, 3.0
- Receives 1 IR for every IR open
- Option to transmit IR in Regular, A/B, A/B/C, A/B/C/D profile
- Supports VW, VWH profiles
- No Predefined & Custom power level and noise level.
- Gradient Noise support.

- No 3.1 mode IR configuration
- No IR Sync available in Async mode.
- Tags will open of IR once in every reporting cycles.
- In Async Room change will be notified as per reporting cycle.
- Monitors will not transmits marks in Async mode.

LF:
- Supports only PPM mode LF transmission

Versioning Scheme: 
Orion:
V a.b.c
a - Device type
b - Hardware version
c - firmware version

Pegasus:
V a.b
a - Device type
b - firmware version