# Test Strategy Document – Safety Bracelet – Orion Release

Rev 1.2

20/06/2025

Revision List

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 11/29/2024 | P. Sharma | Document created |
| 1.1 | 12/2/2024 | A Blackwell | Document updated |
| 1.2 | 20/6/2025 | Cheluva GB | Document updated |

Table of Contents

Project/Test Activity	3

Introduction	3

Team	3

Reference Documents	3

Testing Kick-off Checklist	4

Testing Deliverables and Milestones	5

Scope	5

Out of Scope and Limitations	5

Requirements Traceability Matrix/Test Mapping	5

Testing Type & Coverage	6

Tools/Techniques/Installation and Configuration Methods	6

Tools	6

Installation and Configuration Methods	6

Techniques	6

Test Category	7

Test Success Factors	7

Assumptions, Dependencies and Risk	8

Approvals	9

# Project/Test Activity

Safety Bracelet

# Introduction

This document will detail the test strategy for the Orion release of the Safety bracelet. The current iteration of the firmware only includes support for Orion. This document will be a living document and should be updated on a bi-weekly basis

Safety bracelet is a project which uses 900MHz, IR, and LF for Real Time Location Tracking.

# Team

| Role | Name |
| --- | --- |
| QA Manager | Priyanka Sharma |
| Embedded QA Lead | Cheluva GB |
| Embedded QA Engineer | Cheluva GB/Sagar R |
| System Integration and Test QA Lead | Anthony Blackwell |
| QA Engineer | Nick Giaimo |
| Automation QA Lead | Prasad Babu |
| Automation QA Engineer | Prasad Babu / Kavya D |
| HW Development Manager | Lacho Dimitrov |
| HW Development Lead | Bennett Steber |
| Firmware Manager | Chris Cotignola |
| Team Lead Software Engineering | Mathan Kumar |
| Team Lead Embedded Software | Srinivasan Swaminathan |
| Product Manager | Ash Cunningham |
| Project Manager | Shashi Kumar |

# Reference Documents

This section includes all reference documents used during the validation process.

- Requirement Documentation:

  - PRD: Product Requirements Document (NPD004).xlsx

  - TRD: Technical Requirements Document (NPD004)_v2_1_WIP.xlsx

  - Firmware Design Doc: Safety 2.0 Bracelet - Software Engineering - Confluence

- Design Documentation: Manufacturing

- Project schedule with timelines and milestones. This includes when all major Dev and QA timelines start and end:

  - Insert Project link here : Product Details

# Testing Kick-off Checklist

QA will initiate testing only after the following key criteria have been met:

| Criteria | Status (Complete/On-Track/Blocked) |
| --- | --- |
| Requirements: - Requirements and design documents are reviewed by QA. - All questions from the QA team have been addressed by Dev or PM | On-track |
| Timelines: - The QA team has reviewed and agreed on the test timelines ETA during Project Scheduling | On-track |
| Unit Testing: - Unit testing has been completed by the Dev Team before handing over the code to the QA team. | On-track |
| ** Staging Environment (SW Only) - Staging environment has been locked for QA regression testing. No additional changes to be made once QA has started validation | N/A |

** Updates to the Staging Environment should only happen in conjunction with the QA Team if issues are found during the validation phase.

# Testing Deliverables and Milestones

This section will detail all test deliverables and the milestones of the project.

Note: Deliverables and milestones are driven by the overall project schedule.

| Testing Deliverables | Timeline/ETA | Planned ETA | Actual Completion Date |
| --- | --- | --- | --- |
| Embedded: Full test report with results | Every two weeks | 18:06:2025 |  |
| Embedded: Issues reported during testing | N/A |  |  |
| System Integration: Full test report with results | 3 Months |  |  |
| System Integration: Issues reported during testing | N/A |  |  |
| System Integration: Full E2E validation |  |  |  |
| System Integration: Alpha Testing in Newtown | Two weeks |  |  |

# Scope

This section details the scope of testing. This test run will cover the functionality of the Safety bracelet in Orion as described in the TRD requirements document.

## Embedded Testing

The Embedded Testing Team will cover module level testing of the product based on the features implanted and the availability of the proper supporting infrastructure and documentation for testing. The main scope will be the verification and validation of each feature/module at a module level prior to System Integration. Test cases related to embedded testing can be found from the below TestMO repository.

Repository - Embedded Test Validation - Testmo

## System Integration

The System Integration and Test team will perform a full system level integration test on safety bracelets and core server. This includes the Orion Server, Orion Star, Repeaters, and Pulse.

As part of the System Integration testing, the team will also be performing Alpha testing in the Newtown office with a minimum of 100 devices throughout the office. This testing will provide a baseline of the functionality spread across the an environment designed to simulate the customer site. This alpha testing will also include the team injecting RF noise in certain areas to simulate high traffic areas of a hospital to confirm the bracelets functionality with C-Star. This Alpha testing will utilize the legacy software application Activate. This Alpha testing is expected to take place over a two week period.

# Out of Scope and Limitations

This section details items that are either out of scope of validation or limitations.

- Verification and validation of other RTLS devices.

- The Mobile Application is not currently scheduled to be updated to support the bracelet. This will create some limitations on the validation process.

# Requirements Traceability Matrix/Test Mapping

This section details the requirements traceability of our validation. All TRD items must have

Test MO Run Link: AndromedaNewTCs - Embedded Test Validation - Testmo

| Test Mapping | Requirements | New Features | Test Cases (Link) | Test Type |
| --- | --- | --- | --- | --- |
| Yes | 63 Stories | Yes |  | Manual/Automation |

# Testing Type & Coverage

This section details the types of testing and the percentage of coverage for each type.

| Testing Type | % of Coverage |
| --- | --- |
| Embedded Manual | 100% |
| Embedded Automation | 0% |
| Embedded Semi-Automation | 0% |
| SI&T Manual | 100% |
| SI&T Automation | 0% |
| SI&T Semi-Automation | 0% |

# Tools/Techniques/Installation and Configuration Methods

## Tools

This section details the tools/equipment used to validate the release.

| Item Model Number | Item Serial Number | Calibration Date (If Applicable) |
| --- | --- | --- |
| Joulescope | JS 220 rev E - 003155 | Factory calibrated |
| Logic Analyzer |  |  |

## Installation and Configuration Methods

This section detail the software versions used to validate the release.

| Platform | Version |
| --- | --- |
| Windows Server Version | NA |
| CenTrak Core Server Version |  |
| Activate Version | NA |
| RT Platform Version | NA |

## Software Applications

This section will detail the software applications used to validate the release.

| Sno | Application | Version (if applicable) |
| --- | --- | --- |
| 1 | Testmo | N/A |
| 2 | Wireshark | 4.4.0 |
| 3 | Code Composer Studio | 12.5.0 |
| 4 | Logic Analyzer Software | 2.4.14 |
| 5 | Pegasus RF Sniffer Tool | V36.0.0 |

## Techniques

This section will detail special techniques used during the validation process.

- Devices will be programmed manually via the TI CC1352 Launchpad and upgraded OTA via 900MHz, and Cisco BLE.

# Test Category

This section details the different types of test categories performed as part of this release. Each block should have an updated status of:

| Functional | Coverage & Completion Status | Non-Functional | Coverage & Completion Status |
| --- | --- | --- | --- |
| Unit Testing (Dev) | ☒N/A ☐N/P ☐In Progress ☐Done ☐Blocked | Performance Testing | ☒N/A ☐N/P ☐In Progress ☐Done ☐Blocked |
| Embedded Testing | ☐N/A ☐N/P ☒In Progress ☐Done ☐Blocked | Load Testing | ☒N/A ☐N/P ☐In Progress ☐Done ☐Blocked |
| Integration / End to End Testing | ☐N/A ☒N/T ☐In Progress ☐Done ☐Blocked | Stress Testing | ☒N/A ☐N/P ☐In Progress ☐Done ☐Blocked |
| Regression Testing | ☐N/A ☒N/T ☐In Progress ☐Done ☐Blocked |  |  |

# Test Success Factors

This section will be used to document the test success factors such as the number of bugs found during validation, and the number of bugs closed. Bugs are broken into 3 categories.

- Priority 1: Critical Bugs that must be resolved before release.

- Priority 2: Bugs that do not severely impact the functionality of the product, but may be pushed to the following release with approval from PM.

- Priority 3: Bugs that do not impact the functionality of the product and will be planned for a future release.

| Description | Information |
| --- | --- |
| # of Priority 1 Bugs Opened |  |
| # of Priority 2 Bugs Opened |  |
| # of Priority 3 Bugs Opened |  |
| Total Bug Count |  |
| Total # of Priority 1 Bugs Closed |  |
| Total # of Priority 2 Bugs Closed |  |
| Total # of Priority 3 Bugs Closed |  |
| Test case completion rate |  |

# Assumptions, Dependencies and Risk

This section will detail all assumptions, dependencies, and risks that are being taken during validation.

| Statement | Type |
| --- | --- |
| Statement 1 | Risk |
| Statement 2 | Assumption |
| Statement 3 | Dependency |

# Approvals

This section details the approval status of this document. It is up to the QA Manager to ensure that all responsible parties have approved of the Test Strategy document prior to the start of test.

| Role | Name | Approval Status |
| --- | --- | --- |
| QA Manager | Priyanka Sharma |  |
| Embedded QA Lead | Cheluva GB |  |
| Embedded QA Engineer | Cheluva GB / Sagar R |  |
| System Integration and Test QA Lead | Anthony Blackwell |  |
| QA Engineer | Nick Giaimo |  |
| Automation QA Lead | Prasad Babu |  |
| Automation QA Engineer | Prasad Babu / Kavya D |  |
| HW Development Manager | Lacho Dimitrov |  |
| HW Development Lead | Bennett Steber |  |
| FW Development Manager | Chris Cotignola |  |
| FW Development Lead | Srinivasan Swaminathan |  |
| SW Development Lead | Mathankumar Balasubramaniyan |  |
| Product Manager | Ash Cunningham |  |
| Project Manager | Shashi Kumar PR |  |